LAB 06  —  Soluzione Commentata

Virtualizzazione & Overlay

VRF · GRE · IPSec IKEv2 · DMVPN Ph1/Ph2 · LISP · VXLAN

Uso esclusivo del docente — non distribuire agli studenti

# Indice Errori Pre-configurati

| Bug | Part | Router | Dove | Errore |
| --- | --- | --- | --- | --- |
| 1 | 1 | HUB | Loopback2 CUST-B | IP 10.2.2.1 → deve essere 10.2.1.1 |
| 2 | 1 | SP1 | Loopback2 CUST-B | vrf forwarding CUST-B mancante → Lo2 in global table |
| 3 | 2 | SP1 | Tunnel201 CUST-B | tunnel destination 203.0.113.254 → deve essere 192.0.2.254 |
| 4 | 2 | HUB | Tunnel202 CUST-B | tunnel source Ethernet0/0.10 → deve essere Loopback0 |
| 5 | 5 | HUB | Tunnel210 CUST-B | tunnel mode gre multipoint mancante (priorità: trovare per primo) |
| 6 | 5 | SP1 | Tunnel210 CUST-B | ip nhrp network-id 211 → deve essere 210 |
| 7 | 5 | SP2 | Tunnel210 CUST-B | ip nhrp nhs 172.16.210.2 → deve essere 172.16.210.1 |

PART 0  Verifica Underlay  —  nessuna configurazione richiesta

📝  Nota:  L'underlay è pre-caricato. Gli studenti verificano senza modifiche.

▶  ISP / HUB / SP1 / SP2  —  comandi di verifica

! Su ISP: verifica sub-interface e route verso loopback spoke ISP# show ip interface brief ¦ inc Ethernet¦Loopback ISP# show ip route ! Su HUB: verifica underlay e default route HUB# show ip interface brief HUB# show ip route HUB# ping 198.51.100.2 ! SP1 underlay (atteso: !!!!!) HUB# ping 203.0.113.2 ! SP2 underlay (atteso: !!!!!) HUB# ping 198.51.100.254 source loopback0 ! SP1 Lo0 via ISP HUB# ping 203.0.113.254 source loopback0 ! SP2 Lo0 via ISP

Output atteso

HUB# show ip route ¦ inc 0.0.0.0¦192.0.2 S* 0.0.0.0/0 [1/0] via 192.0.2.1 C 192.0.2.0/30 is directly connected, Ethernet0/0.10 L 192.0.2.2/32 is directly connected, Ethernet0/0.10 C 192.0.2.254/32 is directly connected, Loopback0

✅  CHECKPOINT  Tutti i ping underlay riusciti. Default route presente. Lo0 reachable da tutti i router.

PART 1  VRF Definition + LAN  —  CUST-A (studenti) · CUST-B (correzioni)

Soluzione CUST-A — HUB

HUB — VRF CUST-A + Loopback1

vrf definition CUST-A rd 1:1 ! Route Distinguisher — univoco per VRF address-family ipv4 ! abilita IPv4 nella VRF (sintassi moderna) exit-address-family ! interface Loopback1 description !! VRF CUST-A loopback vrf forwarding CUST-A ! PRIMA del comando ip address ip address 10.1.1.1 255.255.255.255

Soluzione CUST-A — SP1

SP1 — VRF CUST-A + Loopback1

vrf definition CUST-A rd 1:2 address-family ipv4 exit-address-family ! interface Loopback1 vrf forwarding CUST-A ip address 10.1.2.1 255.255.255.255

📝  Nota:  SP2 ha VRF CUST-A e Loopback1 già pre-configurati. Verificare con: show ip route vrf CUST-A

Verifica CUST-A — tutti i router

HUB# show ip route vrf CUST-A HUB# show ip vrf interfaces HUB# show vrf

Output atteso

HUB# show ip route vrf CUST-A 10.0.0.0/32 is subnetted, 1 subnets C 10.1.1.1 is directly connected, Loopback1 HUB# show vrf Name Default RD Protocols Interfaces CUST-A 1:1 ipv4 Lo1 CUST-B 2:1 ipv4 Lo2, Tu201, Tu202, Tu210

🔧 BUG-1 — HUB — Loopback2 CUST-B: IP address errato Diagnosi: Diagnosi. HUB# show ip route vrf CUST-B C 10.2.2.1/32 is directly connected, Loopback2 ! SBAGLIATO: dovrebbe essere 10.2.1.1 ! Conflitto: SP1 usa anche 10.2.2.1 → duplicato nella VRF. Fix: Fix — HUB Lo2. HUB(config)# interface Loopback2 HUB(config-if)# ip address 10.2.1.1 255.255.255.255 ! Non serve ri-assegnare vrf forwarding — è già presente.

🔧 BUG-2 — SP1 — Loopback2 CUST-B: vrf forwarding mancante Diagnosi: Diagnosi. SP1# show ip interface Loopback2 Loopback2 is up, line protocol is up VRF: not set ! BUG: dovrebbe essere CUST-B SP1# show ip route vrf CUST-B ! 10.2.2.1 assente — Lo2 è nella global table SP1# show ip route ¦ inc 10.2.2 C 10.2.2.1/32 is directly connected, Loopback2 ! in global — sbagliato. Fix: Fix — SP1 Lo2. SP1(config)# interface Loopback2 SP1(config-if)# vrf forwarding CUST-B ! ATTENZIONE: il comando rimuove automaticamente l'IP address SP1(config-if)# ip address 10.2.2.1 255.255.255.255 ! Ri-assegnare l'IP dopo aver impostato la VRF.

Verifica finale Part 1

HUB# show ip route vrf CUST-B SP1# show ip route vrf CUST-B SP2# show ip route vrf CUST-B

Output atteso

HUB# show ip route vrf CUST-B C 10.2.1.1/32 is directly connected, Loopback2 ! corretto SP1# show ip route vrf CUST-B C 10.2.2.1/32 is directly connected, Loopback2 ! ora in VRF SP2# show ip route vrf CUST-B C 10.2.3.1/32 is directly connected, Loopback2 ! pre-corretto

✅  CHECKPOINT  Tutte le VRF definite. Loopback in VRF corretta su ogni router. CUST-A: 3 /32 connected. CUST-B: 3 /32 connected (dopo fix).

PART 2  GRE Tunnel in VRF  —  CUST-A (studenti) · CUST-B (correzioni)

Soluzione CUST-A — HUB (Tu101 e Tu102)

HUB — Tunnel101 + Tunnel102 VRF CUST-A

interface Tunnel101 description !! GRE CUST-A HUB<->SP1 vrf forwarding CUST-A ip address 172.16.101.1 255.255.255.252 tunnel source Loopback0 ! Lo0 = 192.0.2.254 (stabile) tunnel destination 198.51.100.254 ! SP1 Lo0 (NBMA address) tunnel mode gre ip ! default, esplicito per chiarezza no shutdown ! interface Tunnel102 description !! GRE CUST-A HUB<->SP2 vrf forwarding CUST-A ip address 172.16.102.1 255.255.255.252 tunnel source Loopback0 tunnel destination 203.0.113.254 ! SP2 Lo0 tunnel mode gre ip no shutdown

Soluzione CUST-A — SP1 (Tu101)

SP1 — Tunnel101 VRF CUST-A

interface Tunnel101 description !! GRE CUST-A SP1<->HUB vrf forwarding CUST-A ip address 172.16.101.2 255.255.255.252 tunnel source Loopback0 ! Lo0 = 198.51.100.254 tunnel destination 192.0.2.254 ! HUB Lo0 tunnel mode gre ip no shutdown

📝  Nota:  SP2 ha Tunnel102 VRF CUST-A già pre-configurato con tunnel protection IPSec (reference Part 4). Verificare che line protocol sia UP: show interface Tu102

Verifica CUST-A

Output atteso

HUB# show interface Tunnel101 ¦ inc protocol¦Source¦Destination¦VRF Tunnel101 is up, line protocol is up VRF: CUST-A Tunnel source 192.0.2.254 (Loopback0), destination 198.51.100.254 HUB# ping vrf CUST-A 172.16.101.2 !!!!! (tunnel IP SP1 raggiungibile)

🔧 BUG-3 — SP1 — Tunnel201 CUST-B: tunnel destination errato Diagnosi: Diagnosi. SP1# show interface Tunnel201 Tunnel protocol/transport GRE/IP Tunnel source 198.51.100.254 (Loopback0) Tunnel destination 203.0.113.254 ! BUG: punta a SP2, non a HUB SP1# ping vrf CUST-B 172.16.201.1 ! IP tunnel HUB — atteso: fallisce ...... Fix: Fix — SP1 Tu201. SP1(config)# interface Tunnel201 SP1(config-if)# tunnel destination 192.0.2.254 ! HUB Lo0. Verifica: ping vrf CUST-B 172.16.201.1 → !!!!!

🔧 BUG-4 — HUB — Tunnel202 CUST-B: tunnel source su sub-interface fisica Diagnosi: Diagnosi. HUB# show interface Tunnel202 Tunnel source 192.0.2.2 (Ethernet0/0.10) ! BUG: non è Loopback0 ! Se e0/0.10 flappa, il tunnel cade anche se ISP è raggiungibile ! via altro path. Inconsistente con best practice.. Fix: Fix — HUB Tu202. HUB(config)# interface Tunnel202 HUB(config-if)# tunnel source Loopback0.

Verifica finale Part 2 — tutti i tunnel UP

HUB# show ip interface brief ¦ inc Tunnel SP1# show ip interface brief ¦ inc Tunnel SP2# show ip interface brief ¦ inc Tunnel

Output atteso

HUB# show ip interface brief ¦ inc Tunnel Tunnel101 172.16.101.1 YES manual up up Tunnel102 172.16.102.1 YES manual up up Tunnel201 172.16.201.1 YES manual up up Tunnel202 172.16.202.1 YES manual up up Tunnel210 172.16.210.1 YES manual up up ! (DMVPN — configurato, bug in Part5)

✅  CHECKPOINT  Tutti i tunnel GRE line protocol UP. Ping sugli IP tunnel (172.16.x.x) riusciti. Nessuna route verso i loopback ancora.

PART 3  Routing Statico in VRF  —  CUST-A

Soluzione — HUB

HUB — static VRF CUST-A

! Route verso loopback SP1 (host /32 via Tu101) ip route vrf CUST-A 10.1.2.1 255.255.255.255 Tunnel101 ! Route verso loopback SP2 (host /32 via Tu102) ip route vrf CUST-A 10.1.3.1 255.255.255.255 Tunnel102

Soluzione — SP1

SP1 — static VRF CUST-A

ip route vrf CUST-A 10.1.1.1 255.255.255.255 Tunnel101 ! verso HUB Lo1 ip route vrf CUST-A 10.1.3.1 255.255.255.255 Tunnel101 ! verso SP2 Lo1 — VIA HUB ! Non esiste Tu103 SP1<->SP2. Il traffico SP1->SP2 transita per HUB. ! Questo e' il limite del design P2P — motivazione per DMVPN (Part 5).

Soluzione — SP2

SP2 — static VRF CUST-A

ip route vrf CUST-A 10.1.1.1 255.255.255.255 Tunnel102 ! verso HUB Lo1 ip route vrf CUST-A 10.1.2.1 255.255.255.255 Tunnel102 ! verso SP1 Lo1 — VIA HUB

Verifica e traceroute

HUB# show ip route vrf CUST-A HUB# ping vrf CUST-A 10.1.2.1 source Loopback1 ! HUB->SP1 Lo1 HUB# ping vrf CUST-A 10.1.3.1 source Loopback1 ! HUB->SP2 Lo1 SP1# ping vrf CUST-A 10.1.3.1 source Loopback1 ! SP1->SP2 Lo1 SP1# traceroute vrf CUST-A 10.1.3.1 source Loopback1

Output atteso

SP1# traceroute vrf CUST-A 10.1.3.1 source Loopback1 1 172.16.101.1 [HUB tunnel IP] — traffico passa per HUB 2 10.1.3.1 [SP2 loopback] ! Nota: SP1->SP2 ha 2 hop (via HUB). DMVPN Phase 2 lo ridurra' a 1.

✅  CHECKPOINT  Ping VRF CUST-A funzionanti su tutti i path. Traceroute SP1->SP2 mostra HUB come hop intermedio.

PART 4  IPSec IKEv2  —  tunnel protection  (reference: SP2 pre-configurato)

📝  Nota:  Esaminare SP2 PRIMA di configurare: show crypto ikev2 proposal / policy / profile  —  show crypto ipsec profile

Soluzione completa — HUB

HUB — IKEv2 + IPSec completo

! --- IKEv2 Proposal: algoritmi di cifratura e integrita' --- crypto ikev2 proposal PROP-ENCOR encryption aes-cbc-256 ! AES 256-bit CBC integrity sha256 ! HMAC-SHA256 group 14 ! Diffie-Hellman 2048-bit ! ! --- IKEv2 Policy: lega proposal alla negoziazione IKE --- crypto ikev2 policy POL-ENCOR proposal PROP-ENCOR ! ! --- IKEv2 Keyring: PSK per ogni peer (identificato da IP NBMA/Lo0) --- crypto ikev2 keyring KR-ENCOR peer SP1 address 198.51.100.254 ! Lo0 di SP1 pre-shared-key cisco123 ! peer SP2 address 203.0.113.254 ! Lo0 di SP2 pre-shared-key cisco123 ! ! --- IKEv2 Profile: match peer, autenticazione, keyring --- crypto ikev2 profile PROF-ENCOR match identity remote address 198.51.100.254 255.255.255.255 match identity remote address 203.0.113.254 255.255.255.255 authentication remote pre-share authentication local pre-share keyring local KR-ENCOR ! ! --- Transform-Set: algoritmi per la SA IPSec (Phase 2) --- crypto ipsec transform-set TS-ENCOR esp-aes 256 esp-sha256-hmac mode tunnel ! incapsula intero pacchetto IP originale ! ! --- IPSec Profile: collega transform-set e IKEv2 profile --- ! Applicato all'interfaccia tunnel (VTI — no crypto map, no ACL) crypto ipsec profile IPSEC-PROF set transform-set TS-ENCOR set ikev2-profile PROF-ENCOR ! ! --- Applica protezione ai tunnel esistenti --- interface Tunnel101 tunnel protection ipsec profile IPSEC-PROF ! interface Tunnel102 tunnel protection ipsec profile IPSEC-PROF

Soluzione — SP1

SP1 — IKEv2 + IPSec

! --- Proposal e Policy: identici a HUB e SP2 --- crypto ikev2 proposal PROP-ENCOR encryption aes-cbc-256 integrity sha256 group 14 ! crypto ikev2 policy POL-ENCOR proposal PROP-ENCOR ! ! --- Keyring SP1: solo peer HUB (SP1 non fa da hub per altri spoke) --- crypto ikev2 keyring KR-ENCOR peer HUB address 192.0.2.254 pre-shared-key cisco123 ! crypto ikev2 profile PROF-ENCOR match identity remote address 192.0.2.254 255.255.255.255 authentication remote pre-share authentication local pre-share keyring local KR-ENCOR ! crypto ipsec transform-set TS-ENCOR esp-aes 256 esp-sha256-hmac mode tunnel ! crypto ipsec profile IPSEC-PROF set transform-set TS-ENCOR set ikev2-profile PROF-ENCOR ! interface Tunnel101 tunnel protection ipsec profile IPSEC-PROF

Sequenza diagnostica IKEv2 — prima e dopo il trigger

! PRIMA del ping (nessuna SA attiva): HUB# show crypto ikev2 sa ! (vuoto) HUB# show crypto isakmp sa ! (vuoto) ! TRIGGER — forza negoziazione IKE: HUB# ping vrf CUST-A 10.1.2.1 source Loopback1 repeat 5 ! DOPO il ping — Phase 1 up: HUB# show crypto isakmp sa HUB# show crypto ikev2 sa HUB# show crypto ipsec sa ¦ include encaps¦decaps

Output atteso

HUB# show crypto isakmp sa IPv4 Crypto ISAKMP SA dst src state conn-id status 198.51.100.254 192.0.2.254 QM_IDLE 1001 ACTIVE 203.0.113.254 192.0.2.254 QM_IDLE 1002 ACTIVE ! QM_IDLE = IKE Phase 1 (ISAKMP SA) attiva — Phase 2 pronta ! MM_ACTIVE = Main Mode completato HUB# show crypto ikev2 sa IPv4 Crypto IKEv2 SA Tunnel-id Local Remote fvrf/ivrf Status 1 192.0.2.254/4500 198.51.100.254/4500 none/CUST-A READY 2 192.0.2.254/4500 203.0.113.254/4500 none/CUST-A READY HUB# show crypto ipsec sa ¦ include encaps¦decaps #pkts encaps: 5, #pkts encrypt: 5, #pkts digest: 5 #pkts decaps: 5, #pkts decrypt: 5, #pkts verify: 5 ! Contatori > 0 = traffico cifrato e decifrato correttamente

✅  CHECKPOINT  SA IKEv2 READY. show crypto isakmp sa: QM_IDLE. show crypto ipsec sa: pkts encaps/decaps > 0. Il traffico VRF CUST-A e' protetto da IPSec.

PART 5  DMVPN Phase 1  —  mGRE + NHRP  (CUST-A studenti · CUST-B fix)

Preparazione — shut down tunnel P2P CUST-A

Preparazione — disattivare tunnel P2P CUST-A

! Commentare o shutdown i tunnel P2P (non eliminare — per confronto) HUB(config)# interface Tunnel101 HUB(config-if)# shutdown HUB(config)# interface Tunnel102 HUB(config-if)# shutdown SP1(config)# interface Tunnel101 SP1(config-if)# shutdown ! SP2 Tu102 shutdown (gia' pre-configurato — attenzione: ha tunnel protection) SP2(config)# interface Tunnel102 SP2(config-if)# shutdown ! Rimuovere anche le route statiche CUST-A (ora gestite via DMVPN) HUB(config)# no ip route vrf CUST-A 10.1.2.1 255.255.255.255 Tunnel101 HUB(config)# no ip route vrf CUST-A 10.1.3.1 255.255.255.255 Tunnel102

Soluzione CUST-A — HUB (Tunnel110 mGRE)

HUB — Tunnel110 DMVPN CUST-A

interface Tunnel110 description !! DMVPN CUST-A cloud — HUB vrf forwarding CUST-A ip address 172.16.110.1 255.255.255.0 tunnel source Loopback0 tunnel mode gre multipoint ! mGRE: accetta connessioni da N spoke ip nhrp network-id 110 ! identifica il cloud DMVPN ip nhrp map multicast dynamic ! replica pacchetti multicast verso spoke registrati ip nhrp redirect ! abilita Phase 2 — notifica spoke di shortcut disponibili tunnel protection ipsec profile IPSEC-PROF no shutdown ! ! Route statiche VRF CUST-A tramite interfaccia DMVPN ip route vrf CUST-A 10.1.2.1 255.255.255.255 Tunnel110 ip route vrf CUST-A 10.1.3.1 255.255.255.255 Tunnel110

Soluzione CUST-A — SP1 (Tunnel110)

SP1 — Tunnel110 DMVPN CUST-A

interface Tunnel110 description !! DMVPN CUST-A cloud — SP1 spoke vrf forwarding CUST-A ip address 172.16.110.11 255.255.255.0 tunnel source Loopback0 tunnel mode gre multipoint ip nhrp network-id 110 ip nhrp nhs 172.16.110.1 ! IP tunnel del NHS (HUB) ip nhrp map 172.16.110.1 192.0.2.254 ! mapping statico: tunnel IP -> NBMA IP HUB ip nhrp map multicast 192.0.2.254 ! traffico multicast verso HUB (EIGRP hellos) ip nhrp shortcut ! abilita Phase 2 — installa route NHRP dirette tunnel protection ipsec profile IPSEC-PROF no shutdown ! ip route vrf CUST-A 10.1.1.1 255.255.255.255 Tunnel110 ip route vrf CUST-A 10.1.3.1 255.255.255.255 Tunnel110

Soluzione CUST-A — SP2 (Tunnel110)

SP2 — Tunnel110 DMVPN CUST-A

interface Tunnel110 description !! DMVPN CUST-A cloud — SP2 spoke vrf forwarding CUST-A ip address 172.16.110.12 255.255.255.0 tunnel source Loopback0 tunnel mode gre multipoint ip nhrp network-id 110 ip nhrp nhs 172.16.110.1 ip nhrp map 172.16.110.1 192.0.2.254 ip nhrp map multicast 192.0.2.254 ip nhrp shortcut tunnel protection ipsec profile IPSEC-PROF no shutdown ! ip route vrf CUST-A 10.1.1.1 255.255.255.255 Tunnel110 ip route vrf CUST-A 10.1.2.1 255.255.255.255 Tunnel110

Verifica DMVPN CUST-A — Phase 1

Output atteso

HUB# show dmvpn Legend: Attrb --> S - Static, D - Dynamic, I - Incomplete N - NATed, L - Local, X - No Socket # Ent Peer NBMA Addr Peer Tunnel Add State UpDn Tm Attrb ----- --------------- --------------- ----- -------- ----- 2 198.51.100.254 172.16.110.11 NHRP 00:02:14 D 203.0.113.254 172.16.110.12 NHRP 00:01:47 D ! D = Dynamic (registrazione spoke ricevuta) — Phase 1 OK SP1# show ip nhrp 172.16.110.1/32 via 172.16.110.1 Tunnel110 created 00:03:10, never expire Type: static, Flags: used NBMA address: 192.0.2.254 SP1# traceroute vrf CUST-A 10.1.3.1 source Loopback1 1 172.16.110.1 [HUB] — traffico ancora via HUB (Phase 1) 2 10.1.3.1

🔧 BUG-5 — HUB — Tunnel210 CUST-B: tunnel mode gre point-to-point (default) Diagnosi — trovare per primo (blocca BUG-6 e BUG-7): Diagnosi. HUB# show interface Tunnel210 Tunnel210 is up, line protocol is up Tunnel protocol/transport GRE/IP ! manca 'Multipoint' HUB# show dmvpn # Ent Peer NBMA Addr ... <- 0 peer registrati ! HUB non e' in modalita' multipoint — non accetta registrazioni. Fix: Fix — HUB Tu210. HUB(config)# interface Tunnel210 HUB(config-if)# tunnel mode gre multipoint HUB# show interface Tunnel210 ¦ inc protocol Tunnel protocol/transport GRE/IP, key disabled, sequencing disabled Multipoint: Yes ! corretto.

🔧 BUG-6 — SP1 — Tunnel210 CUST-B: ip nhrp network-id 211 (dovrebbe essere 210) Diagnosi: Diagnosi. SP1# show dmvpn ! 0 peer — SP1 non e' registrato su HUB (cloud id diverso) HUB# show ip nhrp ! SP1 assente dalla tabella NHS. Fix: Fix — SP1 Tu210. SP1(config)# interface Tunnel210 SP1(config-if)# ip nhrp network-id 210.

🔧 BUG-7 — SP2 — Tunnel210 CUST-B: ip nhrp nhs e map errati Diagnosi: Diagnosi. SP2# show ip nhrp ! nessuna entry — registration mai completata SP2# show dmvpn ! 0 peer — SP2 punta a NHS inesistente (172.16.210.2 = IP di SP1, non HUB). Fix: Fix — SP2 Tu210. SP2(config)# interface Tunnel210 SP2(config-if)# no ip nhrp nhs 172.16.210.2 SP2(config-if)# no ip nhrp map 172.16.210.2 203.0.113.254 SP2(config-if)# no ip nhrp map multicast 203.0.113.254 SP2(config-if)# ip nhrp nhs 172.16.210.1 ! HUB tunnel IP SP2(config-if)# ip nhrp map 172.16.210.1 192.0.2.254 ! HUB NBMA SP2(config-if)# ip nhrp map multicast 192.0.2.254.

Output atteso

HUB# show dmvpn (dopo fix BUG-5/6/7 — CUST-B Tu210) # Ent Peer NBMA Addr Peer Tunnel Add State UpDn 2 198.51.100.254 172.16.210.11 NHRP 00:00:45 203.0.113.254 172.16.210.12 NHRP 00:00:32

✅  CHECKPOINT  DMVPN Phase 1 operativo su CUST-A e CUST-B. Tutti gli spoke registrati (show dmvpn: D). Traceroute SP1->SP2 passa per HUB.

PART 6  DMVPN Phase 2  —  Named EIGRP + spoke-to-spoke diretti

Preparazione — rimuovere route statiche CUST-A (sostituite da EIGRP)

Rimozione route statiche CUST-A

! Rimuovere le route statiche aggiunte in Part 5 HUB(config)# no ip route vrf CUST-A 10.1.2.1 255.255.255.255 Tunnel110 HUB(config)# no ip route vrf CUST-A 10.1.3.1 255.255.255.255 Tunnel110 SP1(config)# no ip route vrf CUST-A 10.1.1.1 255.255.255.255 Tunnel110 SP1(config)# no ip route vrf CUST-A 10.1.3.1 255.255.255.255 Tunnel110 SP2(config)# no ip route vrf CUST-A 10.1.1.1 255.255.255.255 Tunnel110 SP2(config)# no ip route vrf CUST-A 10.1.2.1 255.255.255.255 Tunnel110

Abilitare NHRP Phase 2 — redirect e shortcut

NHRP Phase 2 — HUB redirect + Spoke shortcut

! HUB: abilitare redirect (gia' configurato in Part 5 — verificare) HUB(config)# interface Tunnel110 HUB(config-if)# ip nhrp redirect ! ! HUB: disabilitare next-hop-self per EIGRP (necessario per Phase 2) ! Configurato sotto EIGRP — vedi sotto ! ! SP1 e SP2: abilitare shortcut (gia' configurato in Part 5 — verificare) SP1(config)# interface Tunnel110 SP1(config-if)# ip nhrp shortcut SP2(config)# interface Tunnel110 SP2(config-if)# ip nhrp shortcut

Soluzione Named EIGRP — HUB

HUB — Named EIGRP CUST-A AS1 + CUST-B AS2

router eigrp LAB-ENCOR ! address-family ipv4 vrf CUST-A autonomous-system 1 ! af-interface default passive-interface ! default: non annunciare su interfacce non EIGRP exit-af-interface ! af-interface Tunnel110 no passive-interface ! attivare EIGRP su Tu110 no split-horizon ! CRITICO: HUB deve ri-annunciare route spoke->spoke no next-hop-self ! CRITICO: preserva next-hop originale -> NHRP shortcut funziona hello-interval 20 ! DMVPN best practice: hello piu' lungo su WAN hold-time 60 exit-af-interface ! network 172.16.110.0 0.0.0.255 ! DMVPN cloud CUST-A network 10.1.1.0 0.0.0.255 ! Lo1 HUB eigrp router-id 10.1.1.1 exit-address-family ! address-family ipv4 vrf CUST-B autonomous-system 2 ! af-interface Tunnel210 no passive-interface no split-horizon no next-hop-self hello-interval 20 hold-time 60 exit-af-interface ! network 172.16.210.0 0.0.0.255 network 10.2.1.0 0.0.0.255 eigrp router-id 10.2.1.1 exit-address-family

Soluzione Named EIGRP — SP1

SP1 — Named EIGRP (identico schema su SP2 con IP /router-id propri)

router eigrp LAB-ENCOR ! address-family ipv4 vrf CUST-A autonomous-system 1 af-interface Tunnel110 no passive-interface hello-interval 20 hold-time 60 ! split-horizon e next-hop-self: DEFAULT su spoke (non modificare) exit-af-interface network 172.16.110.0 0.0.0.255 network 10.1.2.0 0.0.0.255 eigrp router-id 10.1.2.1 exit-address-family ! address-family ipv4 vrf CUST-B autonomous-system 2 af-interface Tunnel210 no passive-interface hello-interval 20 hold-time 60 exit-af-interface network 172.16.210.0 0.0.0.255 network 10.2.2.0 0.0.0.255 eigrp router-id 10.2.2.1 exit-address-family

📝  Nota:  SP2: stessa struttura di SP1. Cambiare network 10.1.2.0 -> 10.1.3.0 (CUST-A) e 10.2.2.0 -> 10.2.3.0 (CUST-B). Router-id: 10.1.3.1 e 10.2.3.1

Verifica Phase 2 — spoke-to-spoke diretto

! Verifica neighbor EIGRP HUB# show eigrp address-family ipv4 vrf CUST-A neighbors SP1# show eigrp address-family ipv4 vrf CUST-A neighbors ! Verifica routing CUST-A su SP1 (deve vedere prefissi SP2 via EIGRP) SP1# show ip route vrf CUST-A ! Trigger spoke-to-spoke e verifica shortcut SP1# ping vrf CUST-A 10.1.3.1 source Loopback1 repeat 10 SP1# show ip nhrp detail ¦ inc 10.1.3 SP1# traceroute vrf CUST-A 10.1.3.1 source Loopback1

Output atteso

HUB# show eigrp address-family ipv4 vrf CUST-A neighbors EIGRP-IPv4 VR(LAB-ENCOR) Address-Family Neighbors for AS(1) VRF(CUST-A) H Address Interface Hold Uptime SRTT RTO Q Seq 0 172.16.110.11 Tu110 58 00:03:12 12 200 0 7 1 172.16.110.12 Tu110 57 00:02:48 11 200 0 5 SP1# show ip nhrp ¦ inc 10.1.3 10.1.3.1/32 via 172.16.110.12 Tunnel110 created 00:00:08, expire 00:01:52 Type: dynamic, Flags: router nhop rib nho NBMA address: 203.0.113.254 ! dynamic = NHRP shortcut installato dopo redirect HUB SP1# traceroute vrf CUST-A 10.1.3.1 source Loopback1 1 10.1.3.1 <- path DIRETTO, nessun hop HUB ! Phase 2 operativa: SP1 <-> SP2 bypassano HUB

✅  CHECKPOINT  EIGRP neighbors stabili su Tu110 e Tu210. show ip nhrp: entry dynamic per spoke remoti. Traceroute SP1->SP2: 1 solo hop (path diretto). DMVPN Phase 2 operativa.

PART 7  Riferimenti Teorici — DMVPN Phase 3 · LISP · VXLAN

DMVPN Phase 3 — snippet di configurazione

DMVPN Phase 3 — snippet reference

! Phase 3: HUB annuncia summary invece di route specifiche ! Spoke installa host route /32 NHRP solo per destinazioni attive ! HUB Tu110 — aggiungere summary EIGRP (rimuove specifiche verso spoke) HUB(config)# interface Tunnel110 HUB(config-if)# ip summary-address eigrp 1 10.1.0.0 255.255.0.0 ! HUB annuncia 10.1.0.0/16 invece di 10.1.2.1/32 e 10.1.3.1/32 ! Il redirect rimane attivo — quando SP1 colpisce il summary verso SP2, ! HUB invia Traffic Indication con NBMA di SP2. ! SP1 installa /32 NHRP host route e bypassa HUB per i flussi successivi. ! Differenza da Phase 2: ! Phase 2: HUB non altera next-hop (no next-hop-self) -> spoke-to-spoke ! Phase 3: HUB usa summary per scalabilita' -> shortcut on-demand ! Piu' scalabile (spoke non ricevono tutta la tabella di routing)

LISP — componenti chiave (describe)

LISP — reference

! LISP separa identita' (EID) da locazione (RLOC) ! Non configurabile su IOU — solo teoria ! Componenti: ! EID = Endpoint Identifier — chi sei (indirizzo IP del host) ! RLOC = Routing Locator — dove sei (IP del router di bordo) ! MS/MR = Map-Server / Map-Resolver — database EID->RLOC ! ITR = Ingress Tunnel Router — incapsula e fa la richiesta LISP ! ETR = Egress Tunnel Router — decapsula e risponde al MS ! xTR = router che fa sia ITR che ETR (comune in SD-Access) ! Flusso risoluzione: ! 1. ITR riceve pacchetto verso EID ignoto ! 2. ITR invia Map-Request a MR ! 3. MR risponde con RLOC dell'ETR ! 4. ITR incapsula in UDP/4341 verso RLOC ! 5. ETR decapsula e consegna all'EID ! Caso d'uso Cisco: SD-Access control plane ! Catalyst Center usa LISP per mobilita' endpoint nella fabric

VXLAN — componenti chiave (describe)

VXLAN — reference

! VXLAN: estende L2 su L3 tramite incapsulamento UDP ! Non configurabile su IOU — solo teoria ! Componenti: ! VNI = VXLAN Network Identifier (24 bit = ~16M segmenti vs 4096 VLAN) ! VTEP = VXLAN Tunnel Endpoint — incapsula/decapsula frame Ethernet ! Puo' essere uno switch fisico, un hypervisor (vSwitch), un router ! Outer UDP header: porta destinazione 4789 ! Inner frame: Ethernet completo (L2 over L3) ! BUM traffic (Broadcast/Unknown unicast/Multicast): ! Opzione 1: Multicast underlay (scalabile, richiede mcast routing) ! Opzione 2: Ingress Replication (unicast flood — piu' semplice) ! Caso d'uso Cisco: ! Data center overlay (ACI, VXLAN fabric) ! SD-Access data plane: LISP (control) + VXLAN (data) ! Catalyst Center provision: VTEP = switch campus con VXLAN

# Riepilogo Show Commands — Diagnostica Rapida

| Comando | Cosa verifica | Part |
| --- | --- | --- |
| show vrf | Lista VRF e interfacce associate | 1 |
| show ip vrf interfaces | Interfacce per VRF con IP | 1 |
| show ip interface Lo2 | VRF assegnata — verifica BUG-2 | 1 |
| show ip route vrf CUST-A | Tabella routing per-VRF | 1-3-5-6 |
| show interface Tu<N> | Stato, mode, source/dest, VRF | 2-5 |
| show ip interface brief ¦ inc Tunnel | Tutti i tunnel, stato, IP | 2-5 |
| show crypto ikev2 sa | SA IKEv2 (READY = Phase 1 up) | 4 |
| show crypto isakmp sa | QM_IDLE = Phase 1 up (legacy view) | 4 |
| show crypto ipsec sa | SA IPSec, pkts encaps/decaps | 4 |
| show crypto ikev2 proposal | Proposal configurato | 4 |
| show crypto ipsec profile | IPSec profile + transform-set | 4 |
| show dmvpn | Peer registrati, stato NHRP | 5-6 |
| show dmvpn detail | Attributi per spoke, NHRP type | 5-6 |
| show ip nhrp | Mappature statiche e dinamiche | 5-6 |
| show ip nhrp detail | Tipo: static/dynamic, NBMA addr | 6 |
| show eigrp af-interfaces vrf CUST-A | split-horizon e next-hop-self | 6 |
| show eigrp addr-family ipv4 vrf CUST-A neighbors | Adiacenze EIGRP per VRF | 6 |
| ping vrf CUST-A <IP> source lo1 | Ping VRF-aware | tutti |
| traceroute vrf CUST-A <IP> source lo1 | Path tracking VRF-aware | 3-6 |

Fine Soluzione Commentata — LAB 06  |  CCNP ENCOR 350-401 v1.2  |  Uso esclusivo del docente